TOOLBOX TALK - Circular Saw Safety Tips

Tips for safe use:

• Do not work alone.

• Wear PPE (safety glasses and hearing protection).

• Keep loose hair, clothing, jewelry, and anything that could get caught in the saw tied back.

• Before the cut, set the depth of the saw to 1/8" thicker than material.

• Prepare a stable surface to cut material on.

• Keep guard on saw as manufactured.

• Blade guard should move freely to both open and closed position.

• Never cut with a dull blade.

• Always unplug the saw before you change the blade.

• Cut short material from long material first.

• Stand to the side when cutting material. This positions you away from potential kickback.

• Hold the saw with two hands while cutting.

• After your cut, do not let go of the saw while the blade is spinning.

Questions to start discussion:

1. How can this tool hurt you?

2. How could you prevent it from happening?