TOOLBOX TALK – Falling Objects Near Misses Case study 1

Description of incident:

Structure erection work was under progress at a height of around 20 meters. A tor steel rod (1/2” diameter a & 500mm length) was used for erection work.

After used the tor steel rod, kept on beam; suddenly one worker hand touched/hit the tor steel rod & it fell from 20 meters height.

Root Cause of Incident:

- Improper securing of loose objects.

- Restricting the entry of unauthorized personnel into the work zone.

- Poor supervision.

Preventive action:

- Barricading the bottom area and keeping a standby person at the barricading area to prevent unauthorized entry into the barricaded zone.

- Adequate and vigilant supervision.

- The activity shall be performed after implementing the mitigation measures as per HIRAC.

- Communication of job procedure along with the risks and its control measures to the workers in the toolbox talk.

- Doing housekeeping on a daily basis.

- Securing objects with binding wire/rope to the structure.

- Safety net shall be tied below the beam double layered (arresting men and material)

- All loose objects shall be removed from all floors of the technical structure.

- Metallic bin/jute/Gunny bag shall be used to keep loose materials.