| Site Location: |  | Date: |  |
| --- | --- | --- | --- |
| Work Description: |  | Contractor: |  |

This checklist must be completed whenever a worker is required to enter into an excavation of a depth >1.2m/4ft.  The worker can either be on foot or in the cab of a vehicle/machine

Definition:   An excavation is a hole left in the ground as the result of removing material.  A trench is an excavation in which the depth exceeds the width.  For purposes of this document, the term "excavation" will be used for both trenches and other excavations.

| No. | Item | Yes | No | N/A |
| --- | --- | --- | --- | --- |
| 1 | Have all legally required permits been completed and posted? |  |  |  |
| 2 | Do Confined Space Entry requirements apply? If yes, consult and add Appendix 05 06.04.02.1 Confined Space Checklist. |  |  |  |
| 3 | Has the soil type been determined per OH&S regulation? Has appropriate sloping requirements been determined for safe excavation as per OH&S regulations? |  |  |  |
| 4 | What type of worker protection will be in place for workers at entry and egress points? a. Appropriate sloping as required b. Shoring c. Trench Box d. Other |  |  |  |
| 5 | Are ladders properly deployed at entry/egress points? - tied off at top - extend above shoring or box by 1m/3ft - within 8m/26ft of where personnel are working - other |  |  |  |
| 6 | Is worker entry/egress area clear and free of debris? |  |  |  |
| 7 | Will worker be within the "Control Zone" area at any time (other than entry/egress)? "Control Zone " is defined as a lateral distance inside the excavation extending 0.5 x D from the top for firmer soils (1.5 x D for less firm soils) of the excavation. (D = depth of excavation) Provides same level of protection as at the entry/egress point above. |  |  |  |

| No | Item | Yes | No | N/A |
| --- | --- | --- | --- | --- |
| 8 | If answer to No.7 is “yes”, what type of worker protection will be used for workers in this Control Zone? |  |  |  |
| 9 | If shoring is used, has it been designed by a Professional Engineer, and design available on site? |  |  |  |
| 10 | Is shoring system inspected daily (as a minimum)? |  |  |  |
| 11 | Is excavation spoil kept > 1 meter from edge of excavation? |  |  |  |
| 12 | Are rescue procedures complete and on site? - Are all workers familiar with rescue procedures? - Is a watch person required? |  |  |  |

Completed by: _________________________ Position: _____________________ Date: _________

Site Supervisor: ________________________ Date: __________

Note: Where confined space activity is in conjunction with any critical task procedures the applicable procedures and documentation must be used.